# CSS 422: Hardware and Computer Organization

# Homework: Paged Memory & FP & 2’s Comp

# Instructor Rob Nash

| Memory Device | # of Address Lines | # of Data Lines | # of Write Enable Lines (~WE) | # of Chip Select Lines (~CS) |
| --- | --- | --- | --- | --- |
| (1 chip) | 13 | 16 | 1 | 1 |

## Student Name: Ryan Peters

| Processor | # of Address Lines | # of Data Lines | # of Addressable Bins in Address Space | # of Pages of Memory |
| --- | --- | --- | --- | --- |
| (1 mp) | 17 | 32 | ? | 16 |

- How many addressable memory locations are in each memory device? (1pt)

- How many bits of memory are in each device? (1pt)

- Complete the following table of absolute addresses for the pages specified. (3pts)

| Page | Address (Hex) |
| --- | --- |
| Page 0 Starting Address |  |
| Page 0 Ending Address |  |
| Page 1 Starting Address |  |
| Page 1 Ending Address |  |
| … |  |
| … |  |
| Page 7 Starting Address |  |
| Page 7 Ending Address |  |
| … |  |
| … |  |
| Page 15 Starting Address |  |
| Page 15 Ending Address |  |

- What is the total number of memory devices (chips) required to completely fill this processor’s address space? (1pt)

- Does this memory system support byte-level addressing? Note that without more data, you must assume the smaller memory cannot select bytes. Support your conclusion with a sentence or two. (1pt)

| Processor | # of address lines | # of data lines | Byte Addressable? | # of Addressable Pages in Address Space |
| --- | --- | --- | --- | --- |
| (1 mp) | 30 | 32 | Yes; 4 extra lines control writing to bytes (~W0-~W3) | ? |

- A specific microprocessor is described here.  Using that processor’s specifications, divide the address lines into page number bits, offset bits, and byte-selector bits.  (1 pt)

| Memory Device | Capacity | # of Addressable Locations | # of Data Lines |
| --- | --- | --- | --- |
| (1 chip) | 1Mbit | 128K | 8 |

| Page Bits = | Offset Bits = | Byte-Select Bits= |
| --- | --- | --- |

- How many memory devices/chips would you need to completely fill the addressable memory space of this processor?

- Fill in the address ranges in the following table for the pages of memory indicated. (3pts)

| Page | Address (Hex) |
| --- | --- |
| Page 0 Starting Address |  |
| Page 0 Ending Address |  |
| … | … |
| … | … |
| Page 17 Starting Address |  |
| Page 17 Ending Address |  |
| … | … |
| … | … |
| Page 381 Starting Address |  |
| Page 381 Ending Address |  |
| … | … |
| … | … |
| Page 1024 Starting Address |  |
| Page 1024 Ending Address |  |
| … | … |
| … | … |
| Page 1094 Starting Address |  |
| Page 1094 Ending Address |  |
| … | … |

- Consider the control lines for the memory device, as indicated in the table below.  Write the logical boolean equation for a long word write (of 32 bits).

| ~W0 | ~W1 | ~W2 | ~W3 | ~OE | ~CE |
| --- | --- | --- | --- | --- | --- |
| Attached to data lines D0-D7 | Attached to data lines D8-D15 | Attached to data lines D16-D23 | Attached to data lines D24-D31 | If asserted, output data is made available for a read operation. Could be synthesized from ~W0-~W3. | Enables and disables the chip. |

- What is the logical Boolean equation for the control signals to perform a byte-write to address 01 in little endian format?

- What is the logical Boolean equation for the control signals to perform a word-write to address 00 in big endian format?

- What is the logical Boolean equation for the control signals to perform a byte read for address 11?

- Suppose you want to write the value 5 in binary to address 3 in a 4x3 memory device.  The control and data lines are tabulated below; fill in the correct pattern of control and data signals to accomplish this write.

| R (Reset) | A1 | A0 | D2 | D1 | D0 | ~WE |
| --- | --- | --- | --- | --- | --- | --- |

- Suppose you want to read some data from address 1 in an 8x4 memory device.  The control and data lines should be tabulated below; fill in the names of the appropriate address lines, data lines, and control lines using the previous problem as an example.  Note that this device has an active low write-enable and active high reset, just like the previous chip.  Fill in the lines and the correct pattern of signals to accomplish this read, and you may add more columns to the table.

| R (Reset) |  | A? |  | D? |  | … |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Add more columns? |

- Design the memory circuit described in problem 14 using the built-in RAM device provided in Logisim.   Configure the control lines and test your circuit.   Attach a screenshot here and also upload the “.circ” file.   (3 pts)

- Convert the following numbers into 8-bit hexadecimal values. (2pts)

| Number | Binary | Complemented Form | Two’s Complement | Hex |
| --- | --- | --- | --- | --- |
| -102 |  |  |  |  |
| -87 |  |  |  |  |
| -31 |  |  |  |  |

- Add up the first two binary numbers from the previous problem in Two’s complement form.  (4pts)

  - What is the sum, in hex?

  - Is the sign bit a 0 or a 1?

  - Did overflow occur?

  - Code this up in our 68K simulator by storing the values in registers and adding them up.  Attach a screenshot of the output here and submit both your source and listing files.  What happens?

- Using the single-precision floating-point format defined by IEEE, convert the following numbers from decimal to binary.

| Number | Decimal Value | Starting Form Binary (with Mantissa) | Shifted Binary in Scientific Notation (y * 2c) | Sign Bit | Exponent | Mantissa | Hex |
| --- | --- | --- | --- | --- | --- | --- | --- |
| -69/32 |  |  |  |  |  |  |  |
| 13.625 | 13.625 |  |  |  |  |  |  |

- Convert the following IEEE single-precision floats from hex to decimal.  Show each step of your calculations and how you arrived at your sign bit, exponent, mantissa, and your final answer.

- 0x42E48000

- 0xC6F00040